# APAI2026 - LAB04
PULP_NN

Authors: Călin Diaconu, Davide Nadalini, Alberto Dequino, Francesco Conti

Contacts: calin.diaconu2@unibo.it, seyedahmad.mirsalar2@unibo.it, ihor.sachok@unibo.it

Links: GitHub Link (code)

# Summary

- Subject(s):

  - Parallelization on the PULP architecture

  - Matrix multiplication

  - Fully connected layer

  - 2D convolution

  - Profiling code execution

- Programming Language: 	C

- Lab duration: 			3h

- Assignment:

  - Time for delivery: 1 week

  - Submission deadline: Sunday November 8th 2026 (end of day, 23:59)

# How to deliver the assignment

You will deliver ONLY THIS TEXT FILE, no code

- Download this file.

- Fill in the required results.

- Export to PDF format.

- Rename the file to: LAB<number_of_the_lesson>_APAI_<your_name>.pdf

- Use the Virtuale platform to upload ONLY your .pdf file

# LAB STARTS HERE

# 0.1. Access to the local VM

- On the lab’s PCs, open the file explorer and go to This PC, C:/VM_APAI

- Double click on PULP-box.ova

- VirtualBox opens, just click on “Fine”

- Wait for the VM to be imported

- Open the VM with “Avvia”

- Password is ‘pulp’

# 0.2. (ALL) Open Docker Container with VSCode

Option A (recommended): your own laptop

- Requirements: Docker, VSCode and the Dev Containers extension.

- Clone the GitHub repository of today’s lab: git clone https://github.com/EEESlab/APAI26-LAB04-PULP-NN

- cd APAI26-LAB04-PULP-NN

- Open it with VSCode: code .

- Select the pop-up option “Reopen in container” (the first time, this downloads the container image).

- Open a new terminal. The PULP SDK is configured automatically: there is nothing to source.

Option B: lab machines

- Open the VirtualBox VM (see section 0.1), then open a terminal (right click – open a new terminal)

- Go to the APAI-Docker folder: cd APAI-Docker

- Open Docker with VSCode: code .

- Select the pop-up option “Reopen in container”

- Open a new terminal

- Clone the GitHub repository of today’s lab: git clone https://github.com/EEESlab/APAI26-LAB04-PULP-NN

- cd APAI26-LAB04-PULP-NN

- In EVERY new terminal, set up the PULP SDK: source setup-pulp-sdk.sh

Run the code (both options): go into the exercise folder and run make

- cd <exercise_folder>

- make clean all run

- Or, to choose the number of cores (N = 1..8): make clean all run CORES=N

# Task 1: Matrix Multiplication

Matrix sizes: 	N x K   *   K x M   =  N x M

Initial sizes: 		N=32, 	M=16, 	K=16

## 1.0. Setup

- If working in the virtual machine, don’t forget to source the SDK in new terminals: source setup-pulp-sdk.sh

- Go to 01_matmul_parallelization/ folder

- Every time you want to run the code, SAVE your file and write in the terminal:

make clean all run

## 1.1. Speedup and Amdahl’s law

Task Location: 01_matmul_parallelization/cluster.c

Setup:

- CORES=<varying>

- N=32, M=16, K=16

Sub-tasks:

- Enable performance counters. You will find them in the code → uncomment them

- Fill in the table below: Measure the execution cycles, and calculate the speedup w.r.t. a single core.

Tip: for executing with different numbers of cores, use the “CORES” flag.

Example: make clean all run CORES=8

| Cores (#N) | Cycles | Speedup (w.r.t. CORES=1) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 4 |  |  |
| 8 |  |  |

## 1.2. Explore different input sizes

Task Location: 01_matmul_parallelization/cluster.c

Setup:

- CORES=8

- N=<varying>    M=16     K=16

Sub-tasks:

- Activate the necessary counters in the code

- Measure performance of each individual core (Instructions executed, and IPC)

| Matrix size: N | Instructions executed (each individual core) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 4 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  |  |  |
| 81 |  |  |  |  |  |  |  |  |

| Matrix size: N | IPC (each individual core) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 4 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  |  |  |
| 81 |  |  |  |  |  |  |  |  |

Answer the following questions:

Is the workload equally balanced with N=4? Why?

[ANSWER]:

Is the workload equally balanced with N=8? Why?

[ANSWER]:

Is the IPC higher for N=80 or N=81? Why?

[ANSWER]:

Measure the overall performance of the GEMM: Cycles, MACs, MAC/cycle

Tip: Calculate the MACs by hand

| N | MACs | Cycles (same for all cores) | MAC/cycle (same for all cores) |
| --- | --- | --- | --- |
| 4 |  |  |  |
| 8 |  |  |  |
| 80 |  |  |  |
| 81 |  |  |  |

Answer the following question:

Why is the MAC/cycle lower when N=81 than when N=80?

[ANSWER]:

## 1.3. Load stalls & unrolling the MatMul

Task Location:

- 01_matmul_parallelization/cluster.c

- 01_matmul_parallelization/matmuls.c

Setup:

- 8 CORES

- N=32    M=16     K=16

Sub-tasks:

- Enable the performance counters of interest. We want to profile:

  - Execution cycles (total)

  - Number of instructions executed

  - Load stalls (not in the code yet: you must add this counter)

Complete the code where you find /* YOUR CODE HERE */ with the right performance counters

Put your solution below (code)

[ANSWER]:

Tip: Here’s the full list of the performance counters

Ref for the VIRTUAL MACHINE: /rtos/pmsis/pmsis_api/include/pmsis/chips/default.h

Ref for the DOCKER CONTAINER: /app/pulp-sdk/rtos/pmsis/pmsis_api/include/pmsis/chips/default.h

- Implement the missing code in gemm_unroll() (in matmuls.c). Then manually fix the code in order to unroll 2-4-8-16 operations.

- Now, fill in the table, comparing the naive gemm() vs. the unrolled version gemm_unroll()

|  | Stalls | Instructions | Cycles | IPC | MACs | MAC/cycle |
| --- | --- | --- | --- | --- | --- | --- |
| Naive |  |  |  |  |  |  |
| Unrolled 2 |  |  |  |  |  |  |
| Unrolled 4 |  |  |  |  |  |  |
| Unrolled 8 |  |  |  |  |  |  |
| Unrolled 16 |  |  |  |  |  |  |

Answer the following questions:

Compare the stalls of “naive” vs. unrolled (2-4-8). Which has more? Why?

[ANSWER]:

Why does Unrolled 16 have more stalls than Unrolled 8?

[ANSWER]:

# Task 2: Fully Connected Layer

## 2.0. Setup

- If working in the virtual machine, don’t forget to source the SDK in new terminals: source setup-pulp-sdk.sh

- Go to 02_fully_connected/ folder

- Every time you want to run the code, SAVE your file and write in the terminal:

make clean all run

- Layer dimensions default to the values in include/data_allocation.h (CH_IM_IN=1024, CH_IM_OUT=16). You can override them (and the number of cores) from the command line:

make clean all run CORES=N CH_IN=<x> CH_OUT=<y>

- Inputs and weights are generated at runtime and the output is checked against a reference, so any layer size is valid. If your code is correct, the program prints:

FullyConnected layer executed without errors.

## 2.1. MACs: FullyConnected without vs. with SIMD

Implement the SIMD dot product, then compare the number of executed instructions in the normal FullyConnected layer and the SIMD version.

- Calculate the number of MACs by hand

- Use the performance counters to measure the number of executed instructions with the default (non-SIMD) dotp_u8_i8_i32

- Implement dotp_u8_i8_i32_simd in fully_connected.c (see the TASK 2.1 comment there)

- In fully_connected(), switch the call from dotp_u8_i8_i32 to dotp_u8_i8_i32_simd (look for the comment “Task 2.1”)

- Run again and check that the result is still correct (the program must print “FullyConnected layer executed without errors.”), then measure the number of executed instructions again

Hints for dotp_u8_i8_i32_simd:

- Use the v4u (4 × uint8) and v4s (4 × int8) vector types to load 4 inputs and 4 weights at a time.

- Look at the SumDotp4 macro in include/pulp_nn_utils.h: it performs 4 MACs with a single SIMD instruction.

- The vector length is not always a multiple of 4: make sure the leftover elements are also processed.

Note: the number of executed MAC operations is dictated by the size of the fully connected layer, not the way operations are implemented (non-SIMD/SIMD). The formula used to calculate the total number of MAC operations is:

MACs = Channelsin*Channelsout

You can find the FullyConnected layer dimensions in the include/data_allocation.h header file (default values; see 2.0 to override them).

TIP: 8-bit SIMD instructions perform 4 MACs in 1 cycle.

| Cores (#N) | SIMD | Operations (MAC) | Instructions (#N) |
| --- | --- | --- | --- |
| 1 | No |  |  |
| 1 | Yes |  |  |

Answer the following question:

What is the ratio of instructions before/after the new SIMD implementation? Why?

[ANSWER]:

## 2.2. Calculate speedup

Measure the latency of a fully connected layer and fill out the table below.

From the measured latency calculate the performance and speedup relative to the single-core latency.

Note: to calculate the performance you will have to divide the total number of MAC operations by the measured latency.

| Cores (#N) | SIMD | Latency (cycles) | Performance (MAC/cycle) | Speedup w.r.t. #Cores=1 SIMD=No |
| --- | --- | --- | --- | --- |
| 1 | No |  |  | 1.0x |
| 1 | Yes |  |  |  |
| 8 | No |  |  |  |
| 8 | Yes |  |  |  |

## 2.3. Leftovers and alignment

Study what happens to correctness and performance when the number of input channels is not a multiple of 4.

Setup:

- CORES=1, SIMD on (your dotp_u8_i8_i32_simd)

- CH_OUT=16

- CH_IN=1024, 1020, 1023, 1027

Example:

make clean all run CORES=1 CH_IN=1023 CH_OUT=16

Fill in the table below. Calculate the MACs by hand.

| CH_IN | MACs | Instructions (#N) | Cycles | Performance (MAC/cycle) |
| --- | --- | --- | --- | --- |
| 1024 |  |  |  |  |
| 1020 |  |  |  |  |
| 1023 |  |  |  |  |
| 1027 |  |  |  |  |

Answer the following questions:

Is the result still correct for CH_IN=1023 and CH_IN=1027? Which part of your code makes it so?

[ANSWER]:

Compare the MAC/cycle across the four sizes. Why is it lower when CH_IN is not a multiple of 4? (Hint: think about the address where each weight row starts, and about the leftover loop.)

[ANSWER]:

How could the data layout be changed to avoid the problem?

[ANSWER]:

## 2.4. (OPTIONAL) Load imbalance and input-channel parallelization

So far the 8 cores split the output channels among them. Study what happens when there are few output channels, and try splitting the input channels instead.

Setup:

- CORES=8, SIMD on

- CH_IN=1024

- CH_OUT = 16, 12, 8, 4

Part 1: per-core load

In 02_fully_connected/Makefile, uncomment the following line, so that each core prints its own cycle count:

#APP_CFLAGS += -DPER_CORE_PERF

Run the four CH_OUT values and look at the cycles of each core.

How many output channels does each core get with CH_OUT=12? And with CH_OUT=4?

[ANSWER]:

How many cores are idle in each case?

[ANSWER]:

Part 2: split the input channels

- Implement fully_connected_split_in() in fully_connected.c, following the 4 steps of the TASK 2.4 comment.

- In main.c, switch the call to fully_connected_split_in() (look for the comment “Task 2.4”) and check that the result is still correct.

- Comment out APP_CFLAGS += -DPER_CORE_PERF again in the Makefile before measuring the latency.

Fill in the table below for both versions (output-channel split = fully_connected(), input-channel split = fully_connected_split_in()).

| CH_OUT | Latency (cycles) output split | Latency (cycles) input split | MAC/cycle output split | MAC/cycle input split |
| --- | --- | --- | --- | --- |
| 16 |  |  |  |  |
| 12 |  |  |  |  |
| 8 |  |  |  |  |
| 4 |  |  |  |  |

Answer the following questions:

Why is a barrier needed before the reduction?

[ANSWER]:

What does the reduction cost? For which values of CH_OUT does the input split pay off?

[ANSWER]:

# Task 3: Convolution Layer

## 3.0. Setup

- If working in the virtual machine, don’t forget to source the SDK in new terminals: source setup-pulp-sdk.sh

- Go to 03_convolution/ folder

- Every time you want to run the code, SAVE your file and write in the terminal:

make clean all run

## 3.1. Speedup over multiple cores

Measure the latency of a Convolution layer and fill out the table below. From the measured latency calculate the performance and speedup relative to the single-core latency.

Note: to calculate the performance you will have to divide the total number of MAC operations by the measured latency. The formula to calculate the total number of MAC operations is:

MACs = Kernel Height*Kernel Width*Channelsin*Heightout*Widthout*Channelsout

You can find the Convolution layer dimensions in the include/data_allocation.h header file.

| Cores (#N) | MACs | Latency (cycles) | Performance (MAC/cycle) | Speedup w.r.t. #Cores=1 |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 1.0x |
| 2 |  |  |  |  |
| 4 |  |  |  |  |
| 8 |  |  |  |  |

## 3.2. Where does the time go: im2col vs matmul

Measure how the convolution time is split between its two phases.

The convolution kernel works in two phases. First, im2col copies the 3x3xCH_IN input patch of an output pixel into a contiguous buffer (two output pixels at a time). Then, the matmul kernel multiplies this buffer with the weights.

Setup:

- In 03_convolution/Makefile, uncomment the following line (remove the leading #):

#APP_CFLAGS += -DPROFILE_SPLIT

- Each core now prints its im2col cycles and its matmul cycles.

- Run with CORES=1 and CORES=8. Report the values of core 0, together with the total latency of the layer.

Fill in the table below. im2col share (%) = im2col cycles / total latency × 100.

| Cores (#N) | im2col cycles (core 0) | matmul cycles (core 0) | Total latency (cycles) | im2col share (%) |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 8 |  |  |  |  |

Answer the following questions:

What fraction of the time is spent in im2col? Is the im2col parallelized?

[ANSWER]:

Using Amdahl’s law: if you made the matmul 2x faster, what would the overall speedup be?

[ANSWER]:

Why do the im2col and matmul cycles not add up to the total latency?

[ANSWER]:

## 3.3. Non-power-of-2 number of cores

Measure how the convolution scales when the number of cores is not a power of 2.

Setup:

- CORES = 3, 5, 6, 7 (reuse your results from 3.1 for CORES = 1, 4 and 8)

- Measure the latency with PROFILE_SPLIT commented out, as in 3.1.

- Then enable PROFILE_SPLIT to count how many cores do work: idle cores report 0 cycles.

Example:

make clean all run CORES=6

| Cores (#N) | Latency (cycles) | Speedup w.r.t. #Cores=1 | Cores doing work (#N) |
| --- | --- | --- | --- |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |

Answer the following questions:

Why is the latency with 6 cores almost the same as with 4?

[ANSWER]:

Find the reason in pulp_nn_conv_u8_u8_i8.c (look at how Log2Core and chunk are computed). How would you change the work split so that all cores get work?

[ANSWER]: